/******  BOSS RC-1 FADE (P. GIORGIO 2020 - Covid-19 time)

- Fade function

- Rotary Encoder Volume

- RGB LED for volume visual control

- Integrated Stop Push-Button

*****/

#include <EEPROM.h>

/***********************MCP42XXX Commands************************/

#include <SPI.h>

const int CS_PIN = 10;             // set pin 10 as the slave select (SS) for the digital Potenziometer MCP42100

int potValue = EEPROM.read(0);     // pot value stored by pressing encoder push button!;                      // stored value of the digital Potenziometer

int pot;                           // value of the digital Potenziometer MCP42100 (255 --> 3.3V. --> min MOD)

const int POT0_SEL = 0x11;         // command to the digital potenziometer 0 pin(8-9-10)

const int POT1_SEL = 0x12;         // command to the digital potenziometer 1 pin(5-6-7)

const int BOTH_POT_SEL = 0x13;     // command to the digital potenziometers 0+1 pin(5-6-7)+(8-9-10)

/***********************MCP42XXX Commands************************/

int timeFade = 150;     // time of Fade out

int fade = 0;           // fade counter

int potFade = potValue; // starting value when FADE with external switch

float lenghtPush = 0;   // how long the Foot Switch has been pressed

int shortPush = 100;    // short press -> STOP

int longPush = 450;     // long press  -> FADE function

static int pinA = 2;            // First hardware interrupt pin is digital pin 2  -> Encoder signal A   ->

static int pinB = 3;            // Second hardware interrupt pin is digital pin 3 -> Encoder signal B   ->

static int encoderButton = 4;   // Encoder Push Button D4                                               ->  for all the INPUT the COMMON is on the GND

int encoderPos = potValue;      // to set at the same value stored in the EEPROM

int previousEncoderPos = 0;

volatile byte pinALast = HIGH;

volatile byte stateA = HIGH;

int encoderButtonState = 0;     // read the encoderButton

static int reedStop = 7;        // output to Reed (NO) in parallel with tip of STOP jack and GND                                      ->

const int fadeRing = 8;         // input (diode) from ring jack

const int stopTip = 12;         // input (diode) from tip jack

int stopState = 0;

int ringState = 0;

int stopCount = 0;

int noExt = 1;                  // external STOP switch not connected (default)

int noRing = 1;                 // external RING switch not connected (default) - UNDO/REDO

int Count = 0;

int inter = 0;

const int ledRed = 5;           // RED   1) STOP pressed

const int ledGreen = 9;         // GREEN 1) Ready to FADE 2) after STOP+Clean

const int ledBlue = 6;          // BLUE  1) Fade-in-progress

void setup() {

// Serial.begin(9600);

pinMode(CS_PIN, OUTPUT);     // set the CS_PIN as an output:

SPI.begin();                 // initialize SPI:

pinMode(encoderButton, INPUT_PULLUP); // set Encoder Push Button

pinMode(pinA, INPUT_PULLUP);          // set pinA encoder CHANNEL A

pinMode(pinB, INPUT_PULLUP);          // set pinB encoder CHANNEL B

pinMode(fadeRing, INPUT_PULLUP);      // set ring input

pinMode(stopTip, INPUT_PULLUP);       // set tip input

pinMode(reedStop, OUTPUT);            // Contact STOP (NO)

stopState = digitalRead(stopTip);

delay(10);

if(stopState == LOW){noExt = 0;}  // check if external STOP switch connected

digitalWrite(reedStop, noExt);

ringState = digitalRead(fadeRing);

delay(10);

if(ringState == LOW){noRing = 0;}  // check if external FADE (RING) switch connected

pinMode(ledBlue, OUTPUT);

pinMode(ledRed, OUTPUT);

pinMode(ledGreen, OUTPUT);

// pinMode(A0, OUTPUT);

// pinMode(A1, OUTPUT);

// pinMode(A2, OUTPUT);

// pinMode(A3, OUTPUT);

// pinMode(A4, OUTPUT);

// pinMode(A5, OUTPUT);

// pinMode(A6, OUTPUT);

// pinMode(A7, OUTPUT);

attachInterrupt(digitalPinToInterrupt(pinB),PinB,RISING);  // set an interrupt on PinB, looking for a RISING edge signal and executing the "PinB" Interrupt Service Routine (below)

attachInterrupt(digitalPinToInterrupt(pinA),PinA,FALLING); // set an interrupt on PinA, looking for a FALLING edge signal and executing the "PinA" Interrupt Service Routine (below)

}

void PinA(){

cli(); //stop interrupts happening before we read pin values

stateA = digitalRead(pinA);

if ((pinALast == HIGH) && (stateA == LOW)) {

if (digitalRead(pinB) == HIGH) {

if(encoderPos <= 21) {encoderPos = 21;}

encoderPos = encoderPos - 21;

} else {

if(encoderPos >= 234) {encoderPos = 234;}

encoderPos = encoderPos + 21;

}

}

pinALast = stateA;

sei(); //restart interrupts

}

void PinB(){

cli(); //stop interrupts happening before we read pin values

stateA = digitalRead(pinA);

if ((pinALast == HIGH) && (stateA == LOW)) {

if (digitalRead(pinB) == HIGH) {

if(encoderPos <= 21) {encoderPos = 21;}

encoderPos = encoderPos - 21;

} else {

if(encoderPos >= 234) {encoderPos = 234;}

encoderPos = encoderPos + 21;

}

}

pinALast = stateA;

sei(); //restart interrupts

}

void DigitalPotWrite(int cmd, int val){

val = constrain(val, 0, 255);         // constrain input value within 0 - 255

digitalWrite(CS_PIN, LOW);            // set the CS pin to low to select the chip

SPI.transfer(cmd);                    // send the command and value via SPI

SPI.transfer(val);                    // send the command and value via SPI

digitalWrite(CS_PIN, HIGH);           // Set the CS pin high to execute the command:

}

void Green(){

analogWrite(ledBlue, 0);

analogWrite(ledRed, 0);

analogWrite(ledGreen, 255);

}

void Blue(int Blue){

analogWrite(ledBlue, Blue);

analogWrite(ledRed, 0);

analogWrite(ledGreen, 0);

}

void Red(){

analogWrite(ledBlue, 0);

analogWrite(ledRed, 255);

analogWrite(ledGreen, 0);

}

void RGB(int Vol){

if(Vol > 127){analogWrite(ledBlue, 500-(Vol*2));}

else{analogWrite(ledBlue, (Vol*3)-100);}

analogWrite(ledRed, (Vol*2)-200);

analogWrite(ledGreen, 255-Vol);

previousEncoderPos = encoderPos;

potFade = encoderPos;

}

void RGBall(int Int){

analogWrite(ledBlue, Int);

analogWrite(ledRed, Int);

analogWrite(ledGreen, Int);

}

// void ledVol(int Vol){

//    digitalWrite(A0, LOW);

//    digitalWrite(A1, LOW);

//    digitalWrite(A2, LOW);

//    digitalWrite(A3, LOW);

//    digitalWrite(A4, LOW);

//    digitalWrite(A5, LOW);

//    digitalWrite(A6, LOW);

//    digitalWrite(A7, LOW);

//

//    switch (Vol){

//      case 0:

//      digitalWrite(A0, HIGH);

//      break;

//         case 1:

//         digitalWrite(A1, HIGH);

//         break;

//            case 2:

//            digitalWrite(A2, HIGH);

//            break;

//               case 3:

//               digitalWrite(A3, HIGH);

//               break;

//                  case 4:

//                  digitalWrite(A4, HIGH);

//                  break;

//                     case 5:

//                      digitalWrite(A5, HIGH);

//                      break;

//                         case 6:

//                         digitalWrite(A6, HIGH);

//                         break;

//                            case 7:

//                            digitalWrite(A7, HIGH);

//                            break;

//                 }

//                       }

void loop(){

stateA = digitalRead(pinA);

if ((pinALast == HIGH) && (stateA == LOW)) {

if (digitalRead(pinB) == HIGH) {

if(encoderPos <= 21) {encoderPos = 21;}

encoderPos = encoderPos - 21;

} else {

if(encoderPos >= 234) {encoderPos = 234;}

encoderPos = encoderPos + 21;

}

}

pinALast = stateA;

// the previous part is the same as the interrupt for reading the encoder: I added again because, for some misterious reason..., this improve the reliability of reading!

pot = encoderPos;

DigitalPotWrite(BOTH_POT_SEL , pot);

if(encoderPos != previousEncoderPos) {RGB(encoderPos);}

// ledVol(encoderPos/32);

while (digitalRead(encoderButton) == LOW ){ delay(100);

lenghtPush = lenghtPush + 100;

if (lenghtPush > longPush)        // ready to FADE

{Blue(255);}                      // Blue LED

if (lenghtPush >= 7000)           // ready to store in EEPROM

{RGBall(255);}                 // all LED

if (stopCount == 1) {Green(); digitalWrite(reedStop, LOW);}    // after first STOP

// ready to CLEAN -> green LED

}

digitalWrite(reedStop, noExt);

if (stopCount == 1 && noExt == 0)   // ignore short press if external STOP connected

{stopCount = 0; Green();}

if (stopCount == 1 && lenghtPush >= longPush)

{lenghtPush = 0; stopCount = 0; Green();}

// **************    encoder button pressed for LONG --> Activate  FADE out   ********************

if (lenghtPush >= longPush){

if (lenghtPush >= 7000) {  // press encoder Button for more than 7s

// to store the encoder position in the EEPROM for potValue

EEPROM.update(0, encoderPos);

RGBall(255);

delay(750);

RGBall(0);

delay(750);

RGBall(255);

delay(750);

Green();

goto label;         }

Blue(pot);            // FADE out in progress

// change the resistance on BOTH_POT from encoderPos to min:

for (int pot = encoderPos; pot >= 0; pot--) {

DigitalPotWrite(BOTH_POT_SEL , pot);

Blue(pot);

// ledVol(pot/32);

delay(timeFade);

stopState = digitalRead(stopTip);

delay(10);

if(stopState == HIGH && noExt == 0 || digitalRead(encoderButton) == LOW) {Green();

goto label;}

}

//  Green();

stopCount = 1;

goto label;

}

// **************   encoder button pressed SHORT -> STOP Boss RC-1   **************************

else if(lenghtPush >= shortPush){

stopCount = 1;      // stop pressed

Red();

label:

digitalWrite(reedStop, LOW);

delay(100);

DigitalPotWrite(BOTH_POT_SEL , encoderPos);

digitalWrite(reedStop, noExt);

}

lenghtPush = 0;

// ******************************************************* External FADE switch (RING) *********************

if(noRing == 0){ //  if external FADE (RING) switch connected

ringState = digitalRead(fadeRing);

stopState = digitalRead(stopTip);

delay(175);

if(ringState == HIGH) { Count++; }

if(stopState == HIGH) { Count = 0; Green();}

labelE:

switch (Count) {

case 0:

break;

case 1:

Blue(fade);

// change the resistance on BOTH_POT from potFade to min:

for (fade = potFade; fade >= 0; fade--) {

DigitalPotWrite(BOTH_POT_SEL , fade);

//  ledVol(fade/32);

delay(timeFade);

Blue(fade);

stopState = digitalRead(stopTip);

if(stopState == HIGH) { Count = 0; potFade = encoderPos; goto labelE;}

ringState = digitalRead(fadeRing);

if(ringState == HIGH) { Count = 2; inter = fade; goto labelE;}

}

Green();

potFade = 0; // with external FADE switch it remains at min vol

// until press FADE again -> volume increases progressively (FADE in)

// or press STOP -> volume goes to max

goto labelE;

break;

case 2:

// change the resistance on BOTH_POT from min to potValue:

for (fade = inter; fade <= encoderPos; fade++) {

DigitalPotWrite(BOTH_POT_SEL , fade);

delay(timeFade);

Blue(fade);

//   ledVol(fade/32);

}

Green();

Count = 0;

potFade = encoderPos;

break;

}

}

}